Plug connector overview

| Number | X-pin, colour | Description |
| --- | --- | --- |
| X1081 | 50-pin, natural | Plug connector Airbag diagnostic module |

Pin assignments at plug connector X1081

| Pin | Type | Description / Signal type | Connection / Measuring notes |
| --- | --- | --- | --- |
| 1 | A | Ignition circuit, driver's belt tensioner | Belt tensioner, driver's side |
| 2 | M | Ignition circuit, driver's belt tensioner | Belt tensioner, driver's side |
| 3 | A | Ignition circuit, passenger's belt tensioner | Belt tensioner, passenger's side |
| 4 | M | Ignition circuit, passenger's belt tensioner | Belt tensioner, passenger's side |
| 5 | E | Voltage supply terminal R | Fuse F45 |
| 6 | M | Ground | Ground connector |
| 7 | A | Warning lamp | Instrument cluster |
| 8 | E | Belt signal | Passenger's seat belt buckle switch |
| 9 | E/A | Diagnostic link TXD | Connector TXD |
| 10 | A | Ignition circuit, driver's airbag | Driver's airbag gas generator |
| 11 | M | Ignition circuit, driver's airbag | Driver's airbag gas generator |
| 12 | E | Belt signal | Driver's seat belt buckle switch |
| 13 | A | Ignition circuit, passenger's airbag | Passenger's airbag gas generator 1 |
| 14 | M | Ignition circuit, passenger's airbag | Passenger's airbag gas generator 1 |
| 15 | E | Seat occupancy protection | Electronic seat control |
| 16 | A | Ignition circuit, driver's door side airbag | Inflator assy for side airbag, driver's door |
| 17 | M | Ignition circuit, driver's door side airbag | Inflator assy for side airbag, driver's door |
| 18 | A | Ignition circuit, passenger's door side airbag | Inflator assy for side airbag, passenger's door |
| 19 | M | Ignition circuit, passenger's door side airbag | Inflator assy for side airbag, passenger's door |
| 20 | A | Signal, side airbag sensor | Left front sensor (airbag) |
| 21 | A | Signal, side airbag sensor | Right front sensor (airbag) |
| 22 | A | Ignition circuit, side airbag, rear left door | Generator, side airbag, rear left door |
| 23 | M | Ignition circuit, side airbag, rear left door | Generator, side airbag, rear left door |
| 24 | A | Ignition circuit, side airbag, rear right door | Generator, side airbag, rear right door |
| 25 | M | Ignition circuit, side airbag, rear right door | Generator, side airbag, rear right door |
| 26 |  | Not used |  |
| 27 |  | Not used |  |
| 28 |  | Not used |  |
| 29 |  | Not used |  |
| 30 |  | Not used |  |
| 31 |  | Not used |  |
| 32 |  | Not used |  |
| 33 | A | Ignition circuit, passenger's airbag | Passenger's airbag gas generator 1 |
| 34 | A | Crash signal MRS | General module |
| 35 |  | Not used |  |
| 36 |  | Not used |  |
| 37 |  | Not used |  |
| 38 |  | Not used |  |
| 39 |  | Not used |  |
| 40 | M | Ignition circuit, passenger's airbag | Passenger's airbag gas generator 1 |
| 41 |  | Not used |  |
| 42 |  | Not used |  |
| 43 |  | Not used |  |
| 44 |  | Not used |  |
| 45 | A | Ignition circuit, ITS airbag, driver's side | Generator, driver airbag head area (ITS) |
| 46 | M | Ignition circuit, ITS airbag, driver's side | Generator, driver airbag head area (ITS) |
| 47 | A | Ignition circuit, ITS airbag, passenger's side | Generator, passenger airbag head area (ITS) |
| 48 | M | Ignition circuit, ITS airbag, passenger's side | Generator, passenger airbag head area (ITS) |
| 49 | A | Ignition circuit, battery safety terminal | Generator, battery safety terminal |
| 50 | M | Ignition circuit, battery safety terminal | Generator, battery safety terminal |