Plug connector overview

| Number | X-pin, colour | Description |
| --- | --- | --- |
| X74 | 30-pin, orange | Plug connector Airbag diagnostic module |

Pin assignments at plug connector X74

| Pin | Type | Description / Signal type | Connection / Measuring notes |
| --- | --- | --- | --- |
| 1 | A | Ignition circuit, driver's belt tensioner | Belt tensioner, driver's side |
| 2 | M | Ignition circuit, driver's belt tensioner | Belt tensioner, driver's side |
| 3 | A | Ignition circuit, passenger's belt tensioner | Belt tensioner, passenger's side |
| 4 | M | Ignition circuit, passenger's belt tensioner | Belt tensioner, passenger's side |
| 5 | E | Voltage supply terminal R | Fuse F42 |
| 6 | M | Ground | Ground point |
| 7 | A | Warning lamp | Instrument cluster |
| 8 |  | Not used |  |
| 9 | E/A | Diagnostic link TXD | Diagnostic connector |
| 10 | A | Ignition circuit, driver's airbag | Driver's airbag gas generator |
| 11 | M | Ignition circuit, driver's airbag | Driver's airbag gas generator |
| 12 |  | Not used |  |
| 13 | A | Ignition circuit, passenger's airbag | Passenger's airbag gas generator 1 |
| 14 | M | Ignition circuit, passenger's airbag | Passenger's airbag gas generator 1 |
| 15 | E | Seat occupancy protection | Electronic seat control |
| 16 |  | Not used |  |
| 17 |  | Not used |  |
| 18 |  | Not used |  |
| 19 |  | Not used |  |
| 20 |  | Not used |  |
| 21 |  | Not used |  |
| 22 |  | Not used |  |
| 23 |  | Not used |  |
| 24 |  | Not used |  |
| 25 |  | Not used |  |
| 26 |  | Not used |  |
| 27 |  | Not used |  |
| 28 |  | Not used |  |
| 29 |  | Not used |  |
| 30 |  | Not used |  |