Plug connector overview

| Number | X-pin, colour | Description |
| --- | --- | --- |
| X263 | 10-pin, natural | Plug connector EDC control unit |
| X945 | 26-pin, natural | Plug connector EDC control unit |

Pin assignments at plug connector X263

| Pin | Type | Description / Signal type | Connection / Measuring notes |
| --- | --- | --- | --- |
| 1 | A | Activation, front comfort | Front EDC valves |
| 2 | A | Activation, front normal | Front EDC valves |
| 3 | A | Activation, rear comfort | Rear EDC valves |
| 4 | E | Voltage supply, terminal 15 | Fuse F65 |
| 5 | A | Activation, rear normal | Rear EDC valves |
| 6 | M | Ground | Ground point |
| 7 |  | Not used |  |
| 8 | A | Rear coil voltage | Rear EDC valves |
| 9 | A | Front coil voltage | Front EDC valves |
| 10 |  | Not used |  |

Pin assignments at plug connector X945

| Pin | Type | Description / Signal type | Connection / Measuring notes |
| --- | --- | --- | --- |
| 1 | E | Rear acceleration signal | Acceleration sensor, rear right |
| 2 | A | Switch light | EDC switch |
| 3 | A | up to 1995_08 Not used as of 1995_09 Supply Acceleration sensor, front right | Acceleration sensor, front right |
| 4 | A | up to 1995_08 Not used as of 1995_09 Supply Acceleration sensor, front right | Acceleration sensor, front right |
| 5 | A | up to 1995_08 Not used as of 1995_09 Supply Acceleration sensor, rear right | Acceleration sensor, rear right |
| 6 |  | Not used |  |
| 7 | E | Program setting | EDC switch |
| 8 |  | Not used |  |
| 9 |  | Not used |  |
| 10 | E | Light signal | Light module |
| 11 |  | Not used |  |
| 12 | E | Diagnostic link RXD | Diagnostic connector |
| 13 | E | Wheel speed signal | ABS/ASC control unit |
| 14 | E | Steering angle R | Steering angle sensor |
| 15 | E | Steering angle G | Steering angle sensor |
| 16 |  | Not used |  |
| 17 |  | Not used |  |
| 18 |  | Not used |  |
| 19 | E/A | Diagnostic link TXD | Diagnostic connector |
| 20 | M | Component ground | up to 1995_08 Acceleration sensors as of 1995_09 Acceleration sensor, front right |
| 21 | M | Component ground | Steering angle sensor |
| 22 |  | Not used |  |
| 23 |  | Not used |  |
| 24 | E | Acceleration signal | Acceleration sensor, front right |
| 25 | A | Supply for steering angle sensor | Steering angle sensor |
| 26 | M | up to 1995_08 Not used as of 1995_09 Component ground | Acceleration sensor, rear right |