Connector location views

| Number | X-pin, colour | Description |
| --- | --- | --- |
| X74 | 50-pin, yellow | Plug connector Multiple restraint system control unit |

Pin assignments at plug connector X74

| Pin | Type | Description / Signal type | Connection / Measuring notes |
| --- | --- | --- | --- |
| 1 | A | Positive activation Belt tensioner, driver | Belt tensioner, driver |
| 2 | M | Negative activation Belt tensioner, driver | Belt tensioner, driver |
| 3 | A | Positive activation Belt tensioner, passenger | Belt tensioner, passenger |
| 4 | M | Negative activation Belt tensioner, passenger | Belt tensioner, passenger |
| 5 | E | Terminal R | Fuse F11 |
| 6 | M | Ground | Ground connector |
| 7 | E | Signal, airbag warning LED | Instrument cluster control unit |
| 8 |  | Signal, driver's seat belt contact | Hall-effect sensor, seatbelt buckle |
| 9 | E/A | Signal Diagnostic link TXD | Diagnostic connector |
| 10 | A | Positive activation Airbag, driver's side | Inflator assembly, driver airbag |
| 11 | M | Negative activation Airbag, driver's side | Inflator assembly, driver airbag |
| 12 |  | Signal, front passenger's seat belt contact | Hall-effect sensor, seat belt buckle |
| 13 | A | Positive activation Airbag, front passenger's side | Inflator assembly, passenger airbag stage 1 |
| 14 | M | Negative activation Airbag, front passenger's side | Inflator assembly, passenger airbag stage 1 |
| 15 | A | Signal Seat occupancy recognition | Electronic seat control |
| 16 | A | Positive activation Side airbag, front left | Inflator assy for side airbag, passenger's door |
| 17 | M | Negative activation Side airbag, front left | Inflator assy for side airbag, passenger's door |
| 18 | A | Positive activation Side airbag, front right | Inflator assy for side airbag, driver's door |
| 19 | M | Negative activation Side airbag, front right | Inflator assy for side airbag, driver's door |
| 20 | A | Signal Side airbag, left crash sensor | Sensor for LH side airbag (satellite) |
| 21 | A | Signal Side airbag, right crash sensor | Sensor for RH side airbag (satellite) |
| 22 | A | Positive activation Side airbag, rear left | Inflator assembly, rear left side airbag |
| 23 | M | Negative activation Side airbag, rear left | Inflator assembly, rear left side airbag |
| 24 | A | Positive activation Side airbag, rear right | Inflator assembly, rear right side airbag |
| 25 | M | Negative activation Side airbag, rear right | Inflator assembly, rear right side airbag |
| 26 |  | Not used |  |
| 27 |  | Not used |  |
| 28 |  | Not used |  |
| 29 |  | Not used |  |
| 30 |  | Not used |  |
| 31 |  | Not used |  |
| 32 |  | Not used |  |
| 33 |  | Not used |  |
| 34 | M | Signal, crash sensor | General module control unit |
| 35 |  | Not used |  |
| 36 |  | Not used |  |
| 37 |  | Not used |  |
| 38 |  | Not used |  |
| 39 |  | Not used |  |
| 40 |  | Not used |  |
| 41 |  | Not used |  |
| 42 |  | Not used |  |
| 43 |  | Not used |  |
| 44 |  | Not used |  |
| 45 | A | Positive activation Side airbag, left | Airbag inflator assy, driver's head area |
| 46 | M | Negative activation Side airbag, left | Airbag inflator assy, driver's head area |
| 47 | A | Positive activation Side airbag, right | Airbag inflator assy, passenger's head area |
| 48 | M | Negative activation Side airbag, right | Airbag inflator assy, passenger's head area |
| 49 | A | Battery positive terminal isolation + | Generator, battery safety terminal |
| 50 | M | Battery positive terminal isolation - | Generator, battery safety terminal |